# Theoretical formulation of cantilever beam

Figure 1: General configuration.

For a uniform beam, the Euler-Bernoulli equation in free vibration is:

|  | ρA∂2y∂t2+EI∂4y∂x4=0. | (1) |
| --- | --- | --- |

Given that the system is linear time-invariant (LTI), we can assume a solution of the form:

|  | yx, t=ϕx ejωt. | (2) |
| --- | --- | --- |

This is because complex exponentials are eigenfunctions of any LTI system, i.e., they pass through the system unchanged in shape, only scaled and/or shifted.

Substituting (2) in (1):

|  | ρA ϕx j2ω2ejωt+EI∂4ϕ∂x4ejωt=0, |  |
| --- | --- | --- |
|  | -ρAω2EIϕx+∂4ϕ∂x4=0. |  |

Defining β4=ρAω2/EI, the spatial equation for the mode shapes is:

|  | ∂4ϕ∂x4-β4ϕ=0. | (3) |
| --- | --- | --- |

Assuming a solution of the form ϕx=erx, Eq. (3) becomes:

|  | r4erx-β4erx=0, |  |
| --- | --- | --- |
|  | r4=β4, |  |
|  | r=4β4=β22=±±β2, |  |
|  | r1=β, r2=-β, r2=jβ, r2=-jβ. | (4) |

Since Eq. (1) is linear, the general solution is a linear combination of the four exponential solutions:

|  | ϕx=i=14Cierix=C1 eβx+C2 e-βx+C3 ejβx+C4 e-jβx. | (5) |
| --- | --- | --- |

Eq. (5) has complex exponentials and exponentials with positive and negative arguments. Using Euler’s formula and hyperbolic functions:

|  | eθ=coshθ+sinhθe-θ=coshθ-sinhθejθ=cosθ+jsinθe-jθ=cosθ-jsinθ , |  |
| --- | --- | --- |

the general solution can be expressed in real terms:

|  | C1 eβx+C2 e-βx=C1coshβx+sinhβx+C2coshβx-sinhβx=C1+C2coshβx+C1-C2sinhβx, |  |
| --- | --- | --- |
|  | C3 ejβx+C4 e-jβx=C3cosβx+jsinβx+C4cosβx-jsinβx=C3+C4cosβx+j C3-C4sinβx. |  |

Assuming C3 and C4 are complex conjugates, the constants A=C1+C2, B=C1-C2, C=C3+C4 and D=C3-C4 can be defined and the imaginary constant is cancelled out. Equation (5) is therefore written as:

|  | ϕx=Acoshβx+Bsinhβx+Ccosβx+Dsinβx. | (6) |
| --- | --- | --- |

For a cantilever beam (fixed at x = 0 and free at x = L), the boundary conditions are:

|  | ϕ(0)=0ϕ'0=0ϕ''L=0ϕ'''L=0 . | (7) |
| --- | --- | --- |

Applying the first two boundary conditions from Eq. (7) to Eq. (6):

|  | ϕ0=Acosh0+Bsinh0+Ccos0+Dsin0=0, |  |
| --- | --- | --- |
|  | C=-A, | (8) |
|  | ϕ'0=βA sinh0+Bcosh0-Csin0+Dcos0=0, |  |
|  | D=-B. | (9) |

Substituting Eq. (8) and (9) in (6):

|  | ϕx=Acoshβx-cosβx+Bsinhβx-sinβx | (10) |
| --- | --- | --- |

Applying the two last boundary conditions to Eq. (10), it becomes a linear system of equations in A and B:

|  | ϕ''L=β2AcoshβL+cosβL+BsinhβL+sinβL=0, |  |
| --- | --- | --- |
|  | ϕ'''L=β3AsinhβL-sinβL+BcoshβL+cosβL=0. |  |

In matrix form:

|  | coshβL+cosβLsinhβL+sinβLsinhβL-sinβLcoshβL+cosβLAB=00. |  |
| --- | --- | --- |

For a homogenous linear system, a non-trivial solution exists if and only if the determinant of the matrix is zero. Thus:

|  | coshβL+cosβL2-sinhβL+sinβLsinhβL-sinβL=0, |  |
| --- | --- | --- |
|  | cosh2βL+2coshβLcosβL+cos2βL-sinh2βL+sin2βL=0. | (11) |

Applying the hyperbolic (cosh2θ-sinh2θ=1) and trigonometric (cos2θ+sin2θ=1) identities to Eq. (11), the characteristic equation of the system is obtained:

|  | cosh2βL-sinh2βL→1+cos2βL+sin2βL→1+2coshβLcosβL=0, |  |
| --- | --- | --- |
|  | 2 →1+ 2 →1coshβLcosβL=0, |  |
|  | coshβLcosβL+1=0, β=ρAω2 EI14. | (12) |

Solving Eq. (12) numerically, the first roots are:

|  | β1L=1.88β2L=4.69β3L=7.85β4L=11.0β5L=14.1 . |  |
| --- | --- | --- |

Considering the system parameters from Table 1, the natural frequencies ω=β2EIρA of the system are:

|  | ω1=79.5 rad/s⇒ f1=12.7 Hz ω2=498 rad/s⇒ f2=79.3 Hzω3=1395 rad/s⇒ f3=222 Hzω4=2733 rad/s⇒ f4=435 Hzω5=4519 rad/s⇒ f5=719 Hz | (14) |
| --- | --- | --- |

Table 1: Physical and geometric parameters.

| L (m) | E (Pa) | I (m4) | ρ (kg/m) | A (m²) |
| --- | --- | --- | --- | --- |
| 0.25 | 2.78 x 109 | 7.54 x 10-11 | 1050 | 1.00 x 10-4 |

It should be noted, though, that the formulation so far does not account for the contribution of the actuator’s mass and stiffness to the response of the system. To incorporate it into the characteristic equation, the system can be modeled piecewise as shown in Table 2, where Si is the bending stiffness of each segment, E1, I1, E2, and I2 are the beam’s and the actuator’s elastic moduli and moments of inertia, respectively.

Table 2: Beam discretization.

| Segment | Domain | Si |
| --- | --- | --- |
| 1 | 0≤x≤x1 | E1I1 |
| 2 | x1≤x≤x2 | E1I1+E2I2 |
| 3 | x2≤x≤x3 | E1I1 |
| 4 | x3≤x≤x4 | E1I1 |
| 5 | x4≤x≤x5 | E1I1+E2I2 |
| 6 | x5≤x≤L | E1I1 |

Now the system is discretized into i = 6 segments with shape functions ϕix:

|  | ϕix=Aicoshβx+Bisinhβx+Cicosβx+Disinβx. | (13) |
| --- | --- | --- |

The boundary and continuity conditions are shown in Tables 3 and 4.

Table 3: Boundary conditions.

| Clamped end | Free end |
| --- | --- |
| ϕ10=0 | ϕ6''L=0 |
| ϕ1'0=0 | ϕ6'''L=0 |

Table 4: Continuity conditions.

| Deflection continuity | Slope continuity | Moment continuity | Shear continuity |
| --- | --- | --- | --- |
| ϕ1x1=ϕ2x1 | ϕ1'x1=ϕ2'x1 | ϕ1''x1=ϕ2''x1S2S1 | ϕ1'''x1=ϕ2'''x1S2S1 |
| ϕ2x2=ϕ3x2 | ϕ2'x2=ϕ3'x2 | ϕ2''x2=ϕ3''x2S3S2 | ϕ2'''x2=ϕ3'''x2S3S2 |
| ϕ3x3=ϕ4x3 | ϕ3'x3=ϕ4'x3 | ϕ3''x3=ϕ4''x3S4S3 | S4ϕ4'''x3-S3ϕ3'''x3=maω2ϕ3x3 |
| ϕ4x4=ϕ5x4 | ϕ4'x4=ϕ5'x4 | ϕ4''x4=ϕ5''x4S5S4 | ϕ4'''x4=ϕ5'''x4S5S4 |
| ϕ5x5=ϕ6x5 | ϕ5'x5=ϕ6'x5 | ϕ5''x5=ϕ6''x5S6S5 | ϕ5'''x5=ϕ6'''x5S6S5 |

Differentiating Eq. (13),

|  | ϕi'x=βAisinhβx+Bicoshβx-Cisinβx+Dicosβxϕi''x=β2Aicoshβx+Bisinhβx-Cicosβx-Disinβxϕi'''x=β3Aisinhβx+Bicoshβx+Cisinβx-Dicosβx, |  |
| --- | --- | --- |

the boundary and continuity conditions can be applied, leading to a system of equations of the form:

|  | Mβ∙X=0, |  |
| --- | --- | --- |

where M is a 24 x 24 coefficient matrix and X is the vector of unknowns given by:

| M=M1M2M3M4M5M6M7M8M9, |  |
| --- | --- |
| M1=1010000001010000ch(x1)sh(x1)c(x1)s(x1)-ch(x1)-sh(x1)-c(x1)-s(x1)sh(x1)ch(x1)-s(x1)c(x1)-sh(x1)-ch(x1)s(x1)-c(x1)ch(x1)sh(x1)-c(x1)-s(x1)-S2S1ch(x1)-S2S1sh(x1)S2S1c(x1)S2S1s(x1)sh(x1)ch(x1)s(x1)-c(x1)-S2S1sh(x1)-S2S1ch(x1)-S2S1s(x1)S2S1c(x1)0000ch(x2)sh(x2)c(x2)s(x2)0000sh(x2)ch(x2)-s(x2)c(x2), |  |
| M2=000000000000000000000000000000000000000000000000-ch(x2)-sh(x2)-c(x2)-s(x2)0000-sh(x2)-ch(x2)s(x2)-c(x2)0000, |  |
| M3=M7=0⋯0⋮⋱⋮0⋯08×8, |  |
| M4=0000ch(x2)sh(x2)-c(x2)-s(x2)0000sh(x2)ch(x2)s(x2)-c(x2)000000000000000000000000000000000000000000000000, |  |
| M5=-S3S2ch(x2)-S3S2sh(x2)S3S2c(x2)S3S2s(x2)0000-S3S2sh(x2)-S3S2ch(x2)-S3S2s(x2)S3S2c(x2)0000ch(x3)sh(x3)c(x3)s(x3)-ch(x3)-sh(x3)-c(x3)-s(x3)sh(x3)ch(x3)-s(x3)c(x3)-sh(x3)-ch(x3)s(x3)-c(x3)ch(x3)sh(x3)-c(x3)-s(x3)-S4S3ch(x3)-S4S3sh(x3)S4S3c(x3)S4S3s(x3)-S3β3shx3-maω2chx3-S3β3chx3-maω2shx3-S3β3sx3-maω2cx3S3β3cx3-maω2sx3S4β3shx3S4β3chx3S4β3sx3-S4β3cx30000ch(x4)sh(x4)c(x4)s(x4)0000sh(x4)ch(x4)-s(x4)c(x4), |  |
| M6=000000000000000000000000000000000000000000000000-ch(x4)-sh(x4)-c(x4)-s(x4)0000-sh(x4)-ch(x4)s(x4)-c(x4)0000, |  |
| M8=0000ch(x4)sh(x4)-c(x4)-s(x4)0000sh(x4)ch(x4)s(x4)-c(x4)000000000000000000000000000000000000000000000000, |  |
| M9=-S5S4ch(x4)-S5S4sh(x4)S5S4c(x4)S5S4s(x4)0000-S5S4sh(x4)-S5S4ch(x4)-S5S4s(x4)S5S4c(x4)0000ch(x5)sh(x5)c(x5)s(x5)-ch(x5)-sh(x5)-c(x5)-s(x5)sh(x5)ch(x5)-s(x5)c(x5)-sh(x5)-ch(x5)s(x5)-c(x5)ch(x5)sh(x5)-c(x5)-s(x5)-S6S5ch(x5)-S6S5sh(x5)S6S5c(x5)S6S5s(x5)sh(x5)ch(x5)s(x5)-c(x5)-S6S5sh(x5)-S6S5ch(x5)-S6S5s(x5)S6S5c(x5)0000ch(L)sh(L)-c(L)-s(L)0000sh(L)ch(L)s(L)-c(L), |  |
| chθ=coshβθshθ=sinhβθcθ=cosβθsθ=sinβθ, |  |
| X=X1X2, |  |
| X1T=A1B1C1D1A2B2C2D2A3B3C3D3, |  |
| X2T=A4B4C4D4A5B5C5D5A6B6C6D6. |  |

Solving det(M) = 0 numerically, the roots βi can be found:

(To be continued)