Active vibration control of a cantilever beam using a piezo stack actuator

# Introduction

...

# Modal analysis

Figure 1: General configuration.

For a uniform beam, the Euler-Bernoulli equation in free vibration is:

|  | ρA ∂2y∂t2 + EI ∂4y∂x4 = 0. | (1) |
| --- | --- | --- |

Given that the system is linear time-invariant (LTI), we can assume a solution of the form:

|  | yx, t = ψx ejωt. | (2) |
| --- | --- | --- |

This is because complex exponentials ejωt are eigenfunctions of any LTI system, i.e., they pass through the system unchanged in shape, only scaled and/or shifted.

Substituting (2) in (1):

|  | ρA ψx j2ω2 ejωt + EI ∂4ψ∂x4 ejωt = 0, |  |
| --- | --- | --- |
|  | -ρAω2EIψx + ∂4ψ∂x4 = 0. |  |

Defining β4 = ρAω2/EI, the spatial equation for the mode shapes is:

|  | ∂4ψ∂x4 -β4ψ = 0. | (3) |
| --- | --- | --- |

Assuming a solution of the form ψx = erx, Eq. (3) becomes:

|  | r4erx-β4erx = 0, |  |
| --- | --- | --- |
|  | r4 = β4, |  |
|  | r = 4β4 = β22 = ±±β2, |  |
|  | r1 = β, r2 = – β, r3 = jβ, r4 = – jβ. | (4) |

Since Eq. (1) is linear, the general solution is a linear combination of the four exponential solutions:

|  | ψx = p = 14κperpx = κ1 eβx + κ2 e-βx + κ3 ejβx + κ4 e-jβx. | (5) |
| --- | --- | --- |

Eq. (5) has complex exponentials and exponentials with positive and negative arguments. Using Euler’s formula and hyperbolic functions:

|  | eθ =coshθ+sinhθe-θ =coshθ-sinhθejθ =cosθ + jsinθe-jθ =cosθ – jsinθ, |  |
| --- | --- | --- |

the general solution can be expressed in real terms:

|  | κ1 eβx + κ2 e-βx = κ1coshβx+sinhβx + κ2coshβx-sinhβx = κ1 + κ2coshβx+ κ1-κ2sinhβx, |  |
| --- | --- | --- |
|  | κ3 ejβx + κ4 e-jβx = κ3cosβx+ jsinβx + κ4cosβx-jsinβx = κ3 + κ4cosβx+ j κ3-κ4sinβx. |  |

Assuming κ3 and κ4 are complex conjugates, the constants A = κ1 + κ2, B = κ 1 – κ2, C = κ3 + κ4, D = κ3 – κ4 can be defined and the imaginary constant is cancelled out. Equation (5) is therefore written as:

|  | ψx = Acoshβx+ Bsinhβx+ Ccosβx+ Dsinβx. | (6) |
| --- | --- | --- |

For a cantilever beam (fixed at x = 0 and free at x = L), the boundary conditions are:

|  | ψ(0) = 0ψ'0 = 0ψ''L = 0ψ'''L = 0. | (7) |
| --- | --- | --- |

Applying the first two boundary conditions from Eq. (7) to Eq. (6):

|  | ψ0 = Acosh0 + Bsinh0 + Ccos0 + Dsin0 = 0, |  |
| --- | --- | --- |
|  | C = – A, | (8) |
|  | ψ'0 = βA sinh0 + Bcosh0-Csin0 + Dcos0 = 0, |  |
|  | D = – B. | (9) |

Substituting Eq. (8) and (9) in (6):

|  | ψx = Acoshβx–cosβx+Bsinhβx–sinβx. | (10) |
| --- | --- | --- |

Applying the two last boundary conditions to Eq. (10), it becomes a linear system of equations in A and B:

|  | ψ''L = β2 AcoshβL+cosβL + BsinhβL+sinβL = 0, |  |
| --- | --- | --- |
|  | ψ'''L = β3 AsinhβL–sinβL + BcoshβL+cosβL = 0. |  |

In matrix form:

|  | coshβL+cosβLsinhβL+sinβLsinhβL–sinβLcoshβL+cosβLAB=00. |  |
| --- | --- | --- |

For a homogenous linear system, a non-trivial solution exists if and only if the determinant of the matrix is zero. Thus:

|  | coshβL+cosβL2 – sinhβL+sinβLsinhβL–sinβL = 0, |  |
| --- | --- | --- |
|  | cosh2βL+ 2coshβLcosβL+cos2βL–sinh2βL+sin2βL= 0. | (11) |

Applying the hyperbolic (cosh2θ–sinh2θ=1) and trigonometric (cos2θ+sin2θ=1) identities to Eq. (11), the characteristic equation of the system is obtained:

|  | cosh2βL–sinh2βL→1+cos2βL+sin2βL→1+ 2coshβLcosβL= 0, |  |
| --- | --- | --- |
|  | 2 →1+ 2 →1coshβLcosβL= 0, |  |
|  | coshβLcosβL+ 1 = 0, β=ρAω2 EI14. | (12) |

Solving Eq. (12) numerically, the first roots are:

|  | β1L = 1.88β2L = 4.69β3L = 7.85β4L = 11.0β5L = 14.1. |  |
| --- | --- | --- |

Considering the system parameters from Table 1, the natural frequencies ω'k = β2EIρA and f'k =ω'k2π of the beam are shown in Table 2.

Table 1: Physical and geometric parameters of the beam.

| L (m) | E (Pa) | I (m4) | ρ (kg/m) | A (m²) |
| --- | --- | --- | --- | --- |
| 0.55 | 3.5 x 109 | 6.3 x 10-10 | 850 | 2.1 x 10-4 |

Table 2: Natural frequencies of the beam.

| k | ω'k (rad/s) | f'k (Hz) |
| --- | --- | --- |
| 1 | 40.9 | 6.50 |
| 2 | 256 | 40.7 |
| 3 | 717 | 114 |
| 4 | 1405 | 224 |
| 5 | 2322 | 369 |

It should be noted, though, that the formulation so far does not account for the contribution of the actuator’s mass and stiffness to the response of the system. To incorporate it into the characteristic equation, the system can be modeled piecewise as shown in Table 3, where Si and μi are the bending stiffness and the mass per unit length of each segment i. Ei, Ii, ρi, and Ai are the elastic moduli, moments of inertia, densities, and cross-section areas, with indices 1 and 2 referring to the beam and the actuator supports, respectively.

Table 3: Beam discretization.

| i | Domain | Si | μi |
| --- | --- | --- | --- |
| 1 | 0 ≤ x ≤ x1 | E1I1 | ρ1A1 |
| 2 | x1 ≤ x ≤ x2 | E1I1 + E2I2 | ρ1A1 + ρ2A2 |
| 3 | x2 ≤ x ≤ x3 | E1I1 | ρ1A1 |
| 4 | x3 ≤ x ≤ x4 | E1I1 | ρ1A1 |
| 5 | x4 ≤ x ≤ x5 | E1I1 + E2I2 | ρ1A1 + ρ2A2 |
| 6 | x5 ≤ x ≤ L | E1I1 | ρ1A1 |

The piecewise Euler-Bernoulli equation is:

|  | Sid4ψidx4 – μiω2ψi = 0, | (13) |
| --- | --- | --- |

with the general solution being:

|  | ψix = Aicoshβix + Bisinhβix + Cicosβix + Disinβix, | (14) |
| --- | --- | --- |
|  | βi =μiω2Si14. |  |

Since the beam vibrates as a single system, the frequency ω is common to all segments, thus becoming the main variable of the linear system.

The boundary and continuity conditions are shown in Tables 4 and 5.

Table 4: Boundary conditions.

| Clamped end | Free end |
| --- | --- |
| ψ10 = 0 | ψ''6L = 0 |
| ψ'10 = 0 | ψ'''6L = 0 |

Table 5: Continuity conditions.

| Deflection continuity | Slope continuity | Moment continuity | Shear continuity |
| --- | --- | --- | --- |
| ψ1x1 = ψ2x1 | ψ'1x1 = ψ'2x1 | ψ''1x1 = ψ''2x1S2S1 | ψ'''1x1 = ψ'''2x1S2S1 |
| ψ2x2 = ψ3x2 | ψ'2x2 = ψ'3x2 | ψ''2x2 = ψ''3x2S3S2 | ψ'''2x2 = ψ'''3x2S3S2 |
| ψ3x3 = ψ4x3 | ψ'3x3 = ψ'4x3 | ψ''3x3 = ψ''4x3S4S3 | S4ψ'''4x3 – S3ψ'''3x3 = maω2ψ3x3 |
| ψ4x4 = ψ5x4 | ψ'4x4 = ψ'5x4 | ψ''4x4 = ψ''5x4S5S4 | ψ'''4x4 = ψ'''5x4S5S4 |
| ψ5x5 = ψ6x5 | ψ'5x5 = ψ'6x5 | ψ''5x5 = ψ''6x5S6S5 | ψ'''5x5 = ψ'''6x5S6S5 |

Differentiating Eq. (14),

|  | ψ'1x = βiAisinhβix+ Bicoshβix– Cisinβix+ Dicosβixψ''1x = βi2Aicoshβix+ Bisinhβix– Cicosβix– Disinβixψ'''1 x = βi3Aisinhβix+ Bicoshβix+ Cisinβix– Dicosβix, |  |
| --- | --- | --- |

the boundary and continuity conditions can be applied, leading to a system of equations of the form:

|  | Mω ∙ X = 0, |  |
| --- | --- | --- |

where M is a 24 x 24 coefficient matrix and X is the vector of unknowns given by:

| M = M1M2M3M4M5M6M7M8M9, |  |
| --- | --- |
| M1 =1010000001010000ch1(x1)sh1(x1)c1(x1)s1(x1)-ch2(x1)-sh2(x1)-c2(x1)-s2(x1)sh1(x1)ch1(x1)-s1(x1)c1(x1)-sh2(x1)-ch2(x1)s2(x1)-c2(x1)ch1(x1)sh1(x1)-c1(x1)-s1(x1)-S2S1ch2(x1)-S2S1sh2(x1)S2S1c2(x1)S2S1s2(x1)sh1(x1)ch1(x1)s1(x1)-c1(x1)-S2S1sh2(x1)-S2S1ch2(x1)-S2S1s2(x1)S2S1c2(x1)0000ch2(x2)sh2(x2)c2(x2)s2(x2)0000sh2(x2)ch2(x2)-s2(x2)c2(x2), |  |
| M2 =000000000000000000000000000000000000000000000000-ch3(x2)-sh3(x2)-c3(x2)-s3(x2)0000-sh3(x2)-ch3(x2)s3(x2)-c3(x2)0000, |  |
| M3 = M7 =0⋯0⋮⋱⋮0⋯08×8, |  |
| M4 =0000ch2(x2)sh2(x2)-c2(x2)-s2(x2)0000sh2(x2)ch2(x2)s2(x2)-c2(x2)000000000000000000000000000000000000000000000000, |  |
| M5 =-S3S2ch3(x2)-S3S2sh3(x2)S3S2c3(x2)S3S2s3(x2)0000-S3S2sh3(x2)-S3S2ch3(x2)-S3S2s3(x2)S3S2c3(x2)0000ch3(x3)sh3(x3)c3(x3)s3(x3)-ch4(x3)-sh4(x3)-c4(x3)-s4(x3)sh3(x3)ch3(x3)-s3(x3)c3(x3)-sh4(x3)-ch4(x3)s4(x3)-c4(x3)ch3(x3)sh3(x3)-c3(x3)-s3(x3)-S4S3ch4(x3)-S4S3sh4(x3)S4S3c4(x3)S4S3s4(x3)-S3β33sh3x3-maω2ch3x3-S3β33ch3x3-maω2sh3x3-S3β33s3x3-maω2c3x3S3β33c3x3-maω2s3x3S4β43sh4x3S4β43ch4x3S4β43s4x3-S4β43c4x30000ch4(x4)sh4(x4)c4(x4)s4(x4)0000sh4(x4)ch4(x4)-s4(x4)c4(x4), |  |
| M6 =000000000000000000000000000000000000000000000000-ch5(x4)-sh5(x4)-c5(x4)-s5(x4)0000-sh5(x4)-ch5(x4)s5(x4)-c5(x4)0000, |  |
| M8 =0000ch4(x4)sh4(x4)-c4(x4)-s4(x4)0000sh4(x4)ch4(x4)s4(x4)-c4(x4)000000000000000000000000000000000000000000000000, |  |
| M9 =-S5S4ch5(x4)-S5S4sh5(x4)S5S4c5(x4)S5S4s5(x4)0000-S5S4sh5(x4)-S5S4ch5(x4)-S5S4s5(x4)S5S4c5(x4)0000ch5(x5)sh5(x5)c5(x5)s5(x5)-ch6(x5)-sh6(x5)-c6(x5)-s6(x5)sh5(x5)ch5(x5)-s5(x5)c5(x5)-sh6(x5)-ch6(x5)s6(x5)-c6(x5)ch5(x5)sh5(x5)-c5(x5)-s5(x5)-S6S5ch6(x5)-S6S5sh6(x5)S6S5c6(x5)S6S5s6(x5)sh5(x5)ch5(x5)s5(x5)-c5(x5)-S6S5sh6(x5)-S6S5ch6(x5)-S6S5s6(x5)S6S5c6(x5)0000ch6(L)sh6(L)-c6(L)-s6(L)0000sh6(L)ch6(L)s6(L)-c6(L), |  |
| chiθ =coshβiθshiθ =sinhβiθciθ =cosβiθsiθ =sinβiθ, |  |
| X =X1X2, |  |
| X1T = A1B1C1D1A2B2C2D2A3B3C3D3, |  |
| X2T = A4B4C4D4A5B5C5D5A6B6C6D6. |  |

The characteristic equation is:

|  | detMω = 0. | (15) |
| --- | --- | --- |

A code was developed in Python to solve Eq. (15) numerically and find the angular and linear natural frequencies, ωk and fk = ωk/2π, of the beam-actuator system. Table 6 shows the parameters used, where ma is the actuator’s mass. The positions x1 to x6 and L can be referred to in Figure 1.

Table 6: Physical and geometric parameters of the beam-actuator system.

| E2 (Pa) | I2 (m4) | ρ2 (kg/m) | A2 (m²) | ma (kg) | x1 (m) | x2 (m) | x3 (m) | x4 (m) | x5 (m) | L (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2.0 x 1011 | 1.0 x 10-11 | 7800 | 3.0 x 10-5 | 0.14 | 0.01 | 0.028 | 0.061 | 0.093 | 0.121 | 0.25 |

Table 7 presents the first five natural frequencies and compares them with the values found for the isolated beam (see Table 2).

Table 7: Natural frequencies of the beam-actuator system.

| k | ωk (rad/s) | fk (Hz) | (fk − f'k)/f'k (%) |
| --- | --- | --- | --- |
| 1 | 42.6 | 6.78 | 4 |
| 2 | 253 | 37.4 | –8 |
| 3 | 487 | 77.5 | –32 |
| 4 | 993 | 158 | –29 |
| 5 | 1846 | 294 | –20 |

The presence of the actuator introduced two competing mechanisms: additional bending stiffness, which raises natural frequencies, and additional mass, which lowers them. To comprehend which mechanism dominates in each vibration mode, leading to the variations in frequency shown in Table 7, the mode shapes must be examined.

By applying the natural frequencies and system parameters from Tables 6 and 7 to Eq. (14), the mode shapes ψk,i(x) are obtained. Figure 2 shows the first five mode shapes of the system with and without the actuator normalized by the maximum modal amplitudes for visual comparison.

| (a) Mode 1. | (b) Mode 2. |
| --- | --- |
| (c) Mode 3. | (d) Mode 4. |
| (e) Mode 5. |  |

Figure 2: Comparison of mode shapes with and without the actuator.

...

# System identification

## Mass-normalized mode shapes

The mode shapes ψk(x) are eigenfunctions of Eq. (13) and therefore are defined only up to an arbitrary scaling factor, i.e., any scalar ck multiplied ψk(x) also satisfies the differential eigenvalue problem. To obtain a unique, physically meaningful set of mode shapes, a mass-weighted inner product of the mode shapes can be performed to define scalar constants ck based on system parameters.

In the same way that the dot product of two vectors measures their overlap in Euclidean space, the inner product of two functions,

|  | f(x), g(x) =abfxg(x) dx, |  |
| --- | --- | --- |

measures their overlap in a definite domain a ≤ x ≤ b within the infinite-dimensional Hilbert space.

In the beam-actuator system, the mass-weighted inner product of a mode shape ψk(x) with itself yields:

|  | ψk, ψkμ =i = 16xi - 1xi μiψk,i2x dx + maψk,32x3 = mk, | (16) |
| --- | --- | --- |

where mk is the modal mass of the vibration mode k. The modal mass represents the portion of the system’s mass that contributes to its kinetic energy when vibrating in that mode. By weighting the mass distribution with the modal amplitudes and integrating in space, the modal mass converts the system into a simple, mathematically equivalent single-degree-of-freedom (SDOF) oscillator for that mode.

It should be noted, though, that the modal mass depends on the scaling of ψk. To avoid the dependency on arbitrary scales, the following normalization condition is imposed:

|  | φk, φkμ = 1, | (17) |
| --- | --- | --- |

where φk is a multiple of ψk of the form:

|  | φk(x) = ckψk. | (18) |
| --- | --- | --- |

Substituting Eq. (18) into (17)

|  | ckψk, ckψkμ = 1, |  |
| --- | --- | --- |

Considering the linearity of Eq. (16), ck2 can be factored out:

|  | ck2ψk, ψkμ = 1, |  |
| --- | --- | --- |
|  | ck2 mk = 1, |  |
|  | ck = 1mk , |  |

leading to the final form of the mass-normalized mode shape:

|  | φk(x) = ψk(x)mk. | (19) |
| --- | --- | --- |

## Damping ratios

...

# Control design

...

The system will have an accelerometer at the tip of the beam and a moment actuator near the fixed support. For such a single-input single-output (SISO) system, there are many

## Linear quadratic regulator (LQR)

...